## Option 1:

Stationary object, move hand via robot arm.

Δvslip=vhand- vobj=vhand

Pros:

- vhand captured through ur_rtde. Should be plausible to observe angular velocity at every finger, ωfinger, through direct forward kinematics.

- Repeatable and automatic collection of large amounts of data.

- Can lock/limit DoF to the ones of interest.

Cons:

- Only moves predefined paths.

- Potentially limited in workspace movement.

## Option 2:

Station hand, move objects via human interaction.

Δvslip=vobj- vhand=vobj

Pros:

- More “random” movement through human movement.

- Relatively safe interaction with the shadow hand.

Cons:

- Won’t isolate DoF’s in movement. We can’t guarantee no rotation of object when only looking at  Δvslip

- vobj can be captured through Aruco markers, but can’t be easily expanded to observe angular velocity at every finger, ωfinger